#pragma config(UART_Usage, UART2, uartNotUsed, baudRate4800, IOPins, None, None)

#pragma config(I2C_Usage, I2C1, i2cSensors)

#pragma config(Sensor, in1,    Arm_Angle,      sensorPotentiometer)

#pragma config(Sensor, in2,    Selector,       sensorPotentiometer)

#pragma config(Sensor, I2C_1,  LeftEncoder,    sensorQuadEncoderOnI2CPort,    , AutoAssign )

#pragma config(Sensor, I2C_2,  RightEncoder,   sensorQuadEncoderOnI2CPort,    , AutoAssign )

#pragma config(Motor,  port2,           L_Front,       tmotorVex393_MC29, openLoop, driveLeft, encoderPort, I2C_1)

#pragma config(Motor,  port3,           R_Front,       tmotorVex393_MC29, openLoop, reversed, driveRight, encoderPort, I2C_2)

#pragma config(Motor,  port4,           L_Arm,         tmotorVex393_MC29, openLoop, reversed)

#pragma config(Motor,  port6,           L_Rear,        tmotorVex393_MC29, openLoop)

#pragma config(Motor,  port7,           R_Rear,        tmotorVex393_MC29, openLoop, reversed)

#pragma config(Motor,  port8,           R_Arm,         tmotorVex393_MC29, openLoop)

//*!!Code automatically generated by 'ROBOTC' configuration wizard               !!*//

//VEX COMPETITION INCLUDES

#pragma platform(VEX2)//VEX cortex platform

#pragma competitionControl(Competition)	// Select Download method as "competition"

#include "Vex_Competition_Includes.c"	//do not modify

void pre_auton()	//You must return from this function or the autonomous and usercontrol tasks will not be started.

{

slaveMotor(R_Rear,R_Front);

slaveMotor(L_Rear,L_Front);

slaveMotor(R_Arm,L_Arm);

bStopTasksBetweenModes = false;

}

//drive()

int L_POWER = 0;								//left drive power in drive()

int R_POWER = 0;								//right drive power in drive()

int MAX_POWER = 120;						//maximum power to motor - 127 is max

float DRIVE_SENSITIVITY = 0.8;		//coefficient = decrease joystick sensitivity

int DEADBAND = 12;							//Joystick seldom 0 when off so this is the value to ignore joystick 'noise' below

int ARM_POWER = 0;							//power to apply to arm if needed

float ARM_SENSITIVITY = 0.4;		//slow arm movements

int HOLD_ARM = 18;

//xxxCap() variables - flipArm OK 10-22 (250,50,150,50)

int FLIP_ARM_TIME = 400;		//time lift arm before drive foward

int FLIP_ARM_SPEED = 50;		//power to lift arm

int FLIP_DRIVE_TIME = 150;	//time before driving foward

int FLIP_DRIVE_SPEED = 50;	//drive power

int ARM_MAX = 2100;				//maximum height of arm (3500 max before risk damaging Arm_Angle potentiometer)

int ARM_CAP_HIGH = 2000;		// TBD - value of Arm_Angle potentioneter for placing high caps

int ARM_CAP_LOW = 1380;			//TBD - value of Arm_Angle potentioneter for placing high caps

// autonomous driving variables

int DRIVE_PAUSE = 200;

int BRAKE_TIME = 50;

/*			FUNCTIONS		*/

//TIMED DRIVE

void tDrive(int l, int r, int t){

wait1Msec(DRIVE_PAUSE);

motor[R_Front]=r;

motor[L_Front]=l;

wait1Msec(t);

motor[R_Front]=0;

motor[L_Front]=0;

wait1Msec(BRAKE_TIME);

motor[R_Front]=r *-.5;

motor[L_Front]=l * -.5;

wait1Msec(BRAKE_TIME/2);

motor[R_Front]=0;

motor[L_Front]=0;

}//end tDrive

//DRIVE CONTROL

task drive(){

while(true){

// calculate arcade drive motor[ power

if(abs((vexRT[Ch2] + vexRT[Ch1]))>DEADBAND){

L_POWER =(vexRT[Ch2] + vexRT[Ch1])* DRIVE_SENSITIVITY;

R_POWER =(vexRT[Ch2] - vexRT[Ch1]) * DRIVE_SENSITIVITY;

}

else {

L_POWER = 0;

R_POWER = 0;

}

// limit motor power to MAX_POWER

if(abs(R_POWER) > MAX_POWER)

R_POWER = sgn(R_POWER) * MAX_POWER;

if(abs(L_POWER) > MAX_POWER )

L_POWER = sgn(L_POWER) * MAX_POWER;

motor[L_Front]= L_POWER;

motor[R_Front]=R_POWER;

}//end while

}//end drive

//ARM HEIGHT

task arm(){

while(true){

if(vexRT[Ch3]> DEADBAND && SensorValue(Arm_Angle) < ARM_MAX){//limit height

motor[L_Arm]=vexRT[Ch3] * ARM_SENSITIVITY;

}

else if(vexRT[Ch3] < (DEADBAND * -2) && SensorValue(Arm_Angle) < ARM_MAX){

motor[L_Arm]=vexRT[Ch3] * ARM_SENSITIVITY/4;

}

else if(vexRT[Ch3] < (DEADBAND * -2) && SensorValue(Arm_Angle) > ARM_MAX){

motor[L_Arm]=vexRT[Ch3] * ARM_SENSITIVITY;

}

else{

motor[L_Arm]=ARM_POWER;

}

//if arm is in air - keep from falling

if(SensorValue(Arm_Angle) > 250){

ARM_POWER = HOLD_ARM;

}

else {

ARM_POWER = 0;

}

}//end while

}//end arm

//flip cap

void flipCap(void){

if(SensorValue(Arm_Angle)< 300){

//stop tasks that may interfere with actions

stopTask(arm);

stopTask(drive);

//perform flip by..

motor[L_Arm]=FLIP_ARM_SPEED;	//lift claw

wait1Msec(FLIP_ARM_TIME);		//wait

motor[L_Front]= FLIP_DRIVE_SPEED;	//drive forward to push cap

motor[R_Front]=FLIP_DRIVE_SPEED;

wait1Msec(FLIP_DRIVE_TIME);	//wait

motor[L_Front]= 0;	//stop

motor[R_Front]=0;

motor[L_Arm]=0;

// lower claw TBD (added 10-23)

motor[L_Arm]=FLIP_ARM_SPEED * -0.25 ;	//slowly lower claw

wait1Msec((FLIP_ARM_TIME + FLIP_DRIVE_TIME));	//wait - probably too long

motor[L_Arm]=0;

//re-start tasks

startTask(arm);

startTask(drive);

}//end if

}//end flipCap()

//position arm for placing cap on 36" post

task liftCap(){

while(true){

//lift cap for 36" post

if(vexRT[Btn5U]==1){

stopTask(arm);

while(SensorValue(Arm_Angle) < ARM_CAP_HIGH - 500){

motor[L_Arm]=50;}	//lift claw

while(SensorValue(Arm_Angle) < ARM_CAP_HIGH){

motor[L_Arm]=40;}	//lift claw

motor[L_Arm]=0;

startTask(arm);

}// end btn 5U

//lift cap for 24" post

if(vexRT[Btn5D]==1){

stopTask(arm);

while(SensorValue(Arm_Angle) < ARM_CAP_LOW){

motor[L_Arm]=50;}	//lift claw

motor[L_Arm]=0;

startTask(arm);

}//end 5D

/*		MICRO-ARM MOVEMENT */

//up

if(vexRT[Btn7U]==1){

stopTask(arm);

motor[L_Arm]=35;	//lift claw

wait1Msec(200);

motor[L_Arm]=0;

startTask(arm);}//end 7U

//down

if(vexRT[Btn7D]==1){

stopTask(arm);

motor[L_Arm]=-8;	//lift claw

wait1Msec(250);

motor[L_Arm]=HOLD_ARM;

startTask(arm);}	//end 7D

}//end while

}//end liftCap

void autoA(int direction){  //directi0n = 1 blue, -1 = red side

/*  Autonomous from Position A 	*/

// UNFOLD CLAW & POSITION SO IT DOESN'T DRAG ON MAT

int Arm_Start = SensorValue(Arm_Angle);

while(SensorValue(Arm_Angle)<400){

motor[L_Arm]=75;	//lift  claw

}

while(SensorValue(Arm_Angle)>Arm_Start + 100){

motor[L_Arm]=-35;	//lower  claw

}

while(SensorValue(Arm_Angle)<Arm_Start + 15){

motor[L_Arm]=55;	//lift  claw

}

motor[L_Arm]=10;	//hold claw position

//drive to cap on ball

tDrive(78,70,autoA_fwd); //timed driving distance: Left power, Right power, Time (ms)

//lift cap

while(SensorValue(Arm_Angle) < autoA_liftCap){

motor[L_Arm]=40;}	//lift claw

motor[L_Arm]=15;

tDrive(-50,-50,autoA_backup);	//backup

//turn

tDrive(-65*direction,65*direction,autoA_turn); //timed driving distance: Left power, Right power, Time (ms)

//lower cap

while(SensorValue(Arm_Angle) > Arm_Start){

motor[L_Arm]=-50;}	//lower claw

motor[L_Arm]=0;

// backup

tDrive(-120,-120,autoA_back2platform);

while(SensorValue(Arm_Angle) < 300){

motor[L_Arm]=40;}	//lift claw

motor[L_Arm]=10;// end of autoA  */

}//end autoA

task autonomous(){

if(SensorValue[Selector] <2000){//blue

autoA(1);}

else{autoA(-1);}//red

}

task usercontrol(){

//set-up tasks, sensors etc..

resetMotorEncoder(L_Front);

resetMotorEncoder(R_Front);

startTask(drive);

startTask(arm);

startTask(liftCap);

while (true){

if(vexRT[Btn7L]==1){flipCap();}

/*	MICRO-DRIVE */

if(vexRT[Btn8U]==1){//straight

stopTask(drive);

tDrive(25,25,150);

startTask(drive);}

if(vexRT[Btn8D]==1){	//reverse

stopTask(drive);

tDrive(-25,-25,150);

startTask(drive);}

if(vexRT[Btn8L]==1){	//left

stopTask(drive);

tDrive(-30,30,150);

startTask(drive);}

if(vexRT[Btn8R]==1){	//right

stopTask(drive);

tDrive(30,-30,150);

startTask(drive);}

}//end while

}//end main